Wiring & Interconnect Document

Unit Test / Integration Set  —  CU1, CU2, CU3

Status: Reworked & reviewed against handwritten source (Scanned_20260603-0837.pdf). Items flagged [VERIFY] could not be read with certainty from the scan and should be checked against the original.

# 1.  Scope & Conventions

This document records the point-to-point wiring for a bench test / integration set. Each control unit (CU) is wired to test instrumentation, a comms host and switched power so it can be exercised and measured in isolation. The four sheets describe three units plus a system-level interconnect overview:

- GCF (CU3-P1) — fan control / monitoring.

- INTEGRATED — master connector & loom overview.

- CCM (CU2-P2) — communications module (ECU → PC).

- CAGM (CU1-P1 / CU1-P2) — sensor & power module.

Note on acronyms: GCF, CCM, CAGM, CBO and the CU / J / P designators are project-specific and are not expanded in the source. The descriptions below are inferred from the wiring itself, not from a definition list.

# 2.  Review of the Reproduction (Corrections Log)

A line-by-line comparison of your typed reproduction against the handwritten original surfaced the following. Confirmed corrections have been applied to the tables in Section 4; uncertain readings are flagged for your verification.

| # | Where | Reproduction said | Finding / correction |
| --- | --- | --- | --- |
| 1 | INTEGRATED – loom length | “100m” for every loom | Misread. The source word is “loom” (cable loom), written four times — not a 100 m length. No lengths are given anywhere. Length column removed. |
| 2 | INTEGRATED – loom type | “Arrow loom” / “Standard loom” | Fabricated. The “→” marks before CAGM P1/P2 are pointer arrows, not a loom called “Arrow.” CBO has no “Standard” label. Both removed. |
| 3 | CAGM – RTD terminals | ±1, ±2, ±3 | Likely T1, T2, T3. The paired terminals read clearly as T21/T22/T23, so the companion terminals are most consistently T1/T2/T3; “±” is a misread “T.” [VERIFY] |
| 4 | GCF – fan load | (omitted) | The circled “M” at the AC output is the fan Motor (the GCF load). Added to the table. |
| 5 | CCM – WoW | “3 wires” | The WoW bundle appears to show more than three conductors in the scan. Wire count [VERIFY]. |
| 6 | CCM – AC phases | R, Y, N | Source may show R, Y, B, N (a fourth phase conductor). Captured as R/Y/N; confirm whether a B phase exists. [VERIFY] |
| 7 | CCM – DC destination | “AC/DC Source” | The DC pair most likely returns to the DC Source; the “AC” reading is ambiguous in the scan. [VERIFY] |
| 8 | INTEGRATED – “lnhg” | “lnhg / loop connection” | Underlined word at the foot of the sheet is illegible. “Loop” is a guess. Kept verbatim and flagged. [VERIFY] |
| 9 | CAGM – GUI notes | “(Read/Display)” | Reasonable inference but added text — the source only writes “GUI.” Kept as inference, marked as such. |
| 10 | GCF – unit prefix | “Unit: CU3-P1” | Minor: the source prefixes “All” (or “AU”) before CU3-P1. Noted; meaning unclear. |

# 3.  Block Descriptions

Brief function of each block that appears across the sheets.

## 3.1  Units under test

| Block | Function (inferred from wiring) |
| --- | --- |
| GCF (CU3-P1) | Fan control card. Monitors fan current and a FAULT line (FAN+, FAULT → DMM A3±) and switches a 3-wire AC feed (via 5A relay) that drives the fan Motor (M), plus a DC feed (via 1A relay). |
| CCM (CU2-P2) | Communications module. Carries the ECU serial link (RX±, TX±, GND on connector J4) through a Converter to a PC; also handles a WoW discrete and switched AC/DC supply. |
| CAGM (CU1) | Sensor & actuator module. P1 reads three RTD temperature sensors and a Valve on the DMM; P2 feeds 3-phase AC (through 5A relay, load measured by W) and DC (through 1A relay). |
| CBO | A further module appearing only in the INTEGRATED overview (looms P1, P2). No individual wiring sheet was provided. |

## 3.2  Instrumentation, power & interconnect

| Block | Function (inferred from wiring) |
| --- | --- |
| DMM | Digital multimeter — the measurement instrument where sensor/signal lines terminate (e.g. A3±, T-terminals, valve A1/A2) for current, resistance and voltage checks. |
| PC | Host computer for the serial comms link from the ECU, reached through the Converter. |
| GUI | Operator interface on the PC. Temperature and heater power are read/displayed; the Valve is commanded ON/OFF. |
| Converter | Serial / protocol converter between the ECU differential link (RS-422/485-style RX±, TX±) and the PC. |
| AC Source / DC Source | Programmable supplies feeding the units under test through the protective relays. |
| 5A Relay | Switches/isolates the AC supply line(s) to the unit (rated ~5A). |
| 1A Relay | Switches/isolates the DC supply line(s) to the unit (rated ~1A). |
| M (motor) | The fan motor — the AC load on the GCF sheet (circled “M”). |
| W | Wattmeter / power measurement on the heater load at the CAGM AC output (circled “W”). |
| J0 / J2 / J4 | Main system connectors: J0 = AC power, J2 = DC power, J4 = Comms. J4 is corroborated by the ECU comms connector on the CCM sheet. |
| WoW | Multi-wire discrete signal (commonly “Weight-on-Wheels” / “Wake-on-Wire” — confirm locally). |

# 4.  Corrected Connection Tables

## 4.1  GCF — Unit CU3-P1

| From (signal) | Via | To | Notes |
| --- | --- | --- | --- |
| Controller: FAN+ | Direct | DMM A3+ | Current sense (+) |
| Controller: FAULT | Direct | DMM A3− | Current sense (−); “!” mark in source |
| Power-to-FAN: AC (3 wires) | 5A relay | AC Source | Drives fan Motor (M) |
| Power-to-FAN: DC (2 wires) | 1A relay | DC Source | DC +/− feed |

## 4.2  INTEGRATED — Connectors & Looms

| Signal / Module | Connector / Port | Loom | Notes |
| --- | --- | --- | --- |
| AC Power | J0 | — | Main connector |
| DC Power | J2 | — | Main connector |
| Comms | J4 | — | Matches ECU “J4” on CCM sheet |
| CAGM | P1 | loom |  |
| CAGM | P2 | loom |  |
| CBO | P1 | loom | No individual sheet supplied |
| CBO | P2 | loom |  |
| CU3 (GCF) | P1 | — |  |
| “lnhg” (illegible) | — | — | [VERIFY] reading of foot-of-sheet label |

## 4.3  CCM — Unit CU2-P2

| From (signal) | Via | To | Notes |
| --- | --- | --- | --- |
| ECU J4: RX+ | Converter | PC | Differential serial (RS-422/485-style) |
| ECU J4: RX− | Converter | PC |  |
| ECU J4: TX+ | Converter | PC |  |
| ECU J4: TX− | Converter | PC |  |
| ECU J4: GND | Converter | PC | Signal ground |
| WoW signal | Direct (multi-wire) | — | [VERIFY] wire count |
| AC: R | 5A relay | AC Source out | Phase R |
| AC: Y | 5A relay | AC Source out | Phase Y [VERIFY] (B phase?) |
| AC: N | 5A relay | AC Source out | Neutral |
| DC+ | 1A relay | DC Source | [VERIFY] destination |
| DC− | 1A relay | DC Source |  |

## 4.4  CAGM — Unit CU1-P1 (Sensors → DMM)

| Sensor | Lead | DMM terminal | Notes |
| --- | --- | --- | --- |
| RTD 1 | + | T1 | Temperature sensor [VERIFY: was ±1] |
| RTD 1 | − | T21 | Temperature sensor |
| RTD 2 | + | T2 | [VERIFY: was ±2] |
| RTD 2 | − | T22 |  |
| RTD 3 | + | T3 | [VERIFY: was ±3] |
| RTD 3 | − | T23 |  |
| Valve (4) | A1+, A1−, A2+, A2− | DMM | 4-wire valve control |

## 4.5  CAGM — Unit CU1-P2 (Power via Relays)

| Line | Via | To | Notes |
| --- | --- | --- | --- |
| Ph1 | 5A relay | AC Source |  |
| Ph2 | 5A relay | AC Source |  |
| Ph3 | 5A relay | AC Source |  |
| N | 5A relay | AC Source | Load measured by Wattmeter (W) |
| DC+ | 1A relay | DC Source |  |
| DC− | 1A relay | DC Source |  |

## 4.6  CAGM — GUI Signal Mapping

- Temp. Sensor → GUI (monitor / read)

- Heater (W) → GUI (monitor / read)

- Valve → GUI ON / OFF (control action)

“(monitor / read)” is inferred — the source writes only “GUI” for the sensor and heater rows.

# 5.  Open Items to Verify Against the Original

- RTD terminal labels: confirm T1/T2/T3 vs ±1/±2/±3 (Section 4.4).

- CCM AC phases: confirm R/Y/N vs R/Y/B/N (Section 4.3).

- CCM WoW conductor count (Section 4.3).

- CCM DC return destination — DC Source vs AC Source (Section 4.3).

- INTEGRATED foot-of-sheet label “lnhg” — actual word/meaning (Section 4.2).

- Expansion of the GCF “All / AU” prefix and of the module acronyms (GCF, CCM, CAGM, CBO).